IT-Simplera Institute

Week 2

Submitted By

Afaq Tahir

REG NO

NETB01-8303

Domain

Network Administration

Date Of Submission

10th July, 2026

1. Objectives

- Understand the concepts and importance of Fixed Length Subnet Masking (FLSM) in network design.

- Learn Variable Length Subnet Masking (VLSM) to efficiently allocate IP addresses based on different host requirements.

- Develop the ability to calculate network addresses, subnet masks, broadcast addresses, and usable host ranges for both FLSM and VLSM.

- Understand the concept, purpose, and advantages of Virtual Local Area Networks (VLANs).

- Configure and verify VLANs on Cisco switches using Cisco Packet Tracer and GNS3.

- Configure access ports and trunk ports to enable proper VLAN communication.

- Verify network configurations using Cisco IOS commands and basic connectivity tests.

- Produce professional network documentation, including topology diagrams and technical reports.

2. Fixed Length Subnet Masking (FLSM)

2.1 What is FLSM?

Fixed Length Subnet Masking, commonly referred to as FLSM, is a subnetting technique where a single large network is divided into multiple smaller subnetworks of equal size. The defining characteristic of FLSM is that every subnet created from the parent network gets exactly the same subnet mask, which means each subnet has an identical number of usable IP addresses. This approach is straightforward to implement and easy to understand, which makes it a good starting point for learning subnetting concepts.

The main limitation of FLSM is that it does not take into account the actual number of hosts required by different departments or network segments. For example, if one department needs 100 hosts and another needs only 5, FLSM still assigns the same block size to both — resulting in significant IP address wastage in smaller departments. Despite this limitation, FLSM remains widely used in smaller networks, lab environments, and situations where simplicity is preferred over efficiency.

2.2 How FLSM Works — Calculation Method

To perform FLSM subnetting, the following steps are followed:

- Step 1: Determine the number of subnets required.

- Step 2: Calculate the number of bits to borrow from the host portion: if you need N subnets, borrow enough bits so that 2^n >= N.

- Step 3: Add the borrowed bits to the original prefix to get the new subnet mask.

- Step 4: Calculate the block size: subtract the last octet of the new subnet mask from 256.

- Step 5: List all subnets by incrementing the network address by the block size each time.

- Step 6: For each subnet, identify the Network Address, Subnet Mask, First Usable Host, Last Usable Host, and Broadcast Address.

2.3 FLSM Worksheet — 10 Solved Problems

Problem 1: Network 192.168.1.0/24 — divide into 4 equal subnets

| Subnet | Network Address | Subnet Mask / CIDR | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- | --- |
| Subnet 1 | 192.168.1.0 | 255.255.255.192 /26 | 192.168.1.1 | 192.168.1.62 | 192.168.1.63 |
| Subnet 2 | 192.168.1.64 | 255.255.255.192 /26 | 192.168.1.65 | 192.168.1.126 | 192.168.1.127 |
| Subnet 3 | 192.168.1.128 | 255.255.255.192 /26 | 192.168.1.129 | 192.168.1.190 | 192.168.1.191 |
| Subnet 4 | 192.168.1.192 | 255.255.255.192 /26 | 192.168.1.193 | 192.168.1.254 | 192.168.1.255 |

Problem 2: Network 192.168.2.0/24 — divide into 8 equal subnets

| Subnet | Network Address | Subnet Mask / CIDR | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- | --- |
| Subnet 1 | 192.168.2.0 | 255.255.255.224 /27 | 192.168.2.1 | 192.168.2.30 | 192.168.2.31 |
| Subnet 2 | 192.168.2.32 | 255.255.255.224 /27 | 192.168.2.33 | 192.168.2.62 | 192.168.2.63 |
| Subnet 3 | 192.168.2.64 | 255.255.255.224 /27 | 192.168.2.65 | 192.168.2.94 | 192.168.2.95 |
| Subnet 4 | 192.168.2.96 | 255.255.255.224 /27 | 192.168.2.97 | 192.168.2.126 | 192.168.2.127 |
| Subnet 5 | 192.168.2.128 | 255.255.255.224 /27 | 192.168.2.129 | 192.168.2.158 | 192.168.2.159 |
| Subnet 6 | 192.168.2.160 | 255.255.255.224 /27 | 192.168.2.161 | 192.168.2.190 | 192.168.2.191 |
| Subnet 7 | 192.168.2.192 | 255.255.255.224 /27 | 192.168.2.193 | 192.168.2.222 | 192.168.2.223 |
| Subnet 8 | 192.168.2.224 | 255.255.255.224 /27 | 192.168.2.225 | 192.168.2.254 | 192.168.2.255 |

Problem 3: Network 172.16.0.0/16 — divide into 4 equal subnets

| Subnet | Network Address | Subnet Mask / CIDR | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- | --- |
| Subnet 1 | 172.16.0.0 | 255.255.192.0 /18 | 172.16.0.1 | 172.16.63.254 | 172.16.63.255 |
| Subnet 2 | 172.16.64.0 | 255.255.192.0 /18 | 172.16.64.1 | 172.16.127.254 | 172.16.127.255 |
| Subnet 3 | 172.16.128.0 | 255.255.192.0 /18 | 172.16.128.1 | 172.16.191.254 | 172.16.191.255 |
| Subnet 4 | 172.16.192.0 | 255.255.192.0 /18 | 172.16.192.1 | 172.16.255.254 | 172.16.255.255 |

Problem 4: Network 10.0.0.0/8 — divide into 4 equal subnets

| Subnet | Network Address | Subnet Mask / CIDR | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- | --- |
| Subnet 1 | 10.0.0.0 | 255.192.0.0 /10 | 10.0.0.1 | 10.63.255.254 | 10.63.255.255 |
| Subnet 2 | 10.64.0.0 | 255.192.0.0 /10 | 10.64.0.1 | 10.127.255.254 | 10.127.255.255 |
| Subnet 3 | 10.128.0.0 | 255.192.0.0 /10 | 10.128.0.1 | 10.191.255.254 | 10.191.255.255 |
| Subnet 4 | 10.192.0.0 | 255.192.0.0 /10 | 10.192.0.1 | 10.255.255.254 | 10.255.255.255 |

Problem 5: Network 192.168.10.0/24 — divide into 2 equal subnets

| Subnet | Network Address | Subnet Mask / CIDR | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- | --- |
| Subnet 1 | 192.168.10.0 | 255.255.255.128 /25 | 192.168.10.1 | 192.168.10.126 | 192.168.10.127 |
| Subnet 2 | 192.168.10.128 | 255.255.255.128 /25 | 192.168.10.129 | 192.168.10.254 | 192.168.10.255 |

Problem 6: Network 192.168.20.0/24 — divide into 16 equal subnets

| Subnet | Network Address | Subnet Mask / CIDR | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- | --- |
| Subnet 1 | 192.168.20.0 | 255.255.255.240 /28 | 192.168.20.1 | 192.168.20.14 | 192.168.20.15 |
| Subnet 2 | 192.168.20.16 | 255.255.255.240 /28 | 192.168.20.17 | 192.168.20.30 | 192.168.20.31 |
| Subnet 3 | 192.168.20.32 | 255.255.255.240 /28 | 192.168.20.33 | 192.168.20.46 | 192.168.20.47 |
| Subnet 4 | 192.168.20.48 | 255.255.255.240 /28 | 192.168.20.49 | 192.168.20.62 | 192.168.20.63 |
| Subnet 5 | 192.168.20.64 | 255.255.255.240 /28 | 192.168.20.65 | 192.168.20.78 | 192.168.20.79 |
| Subnet 6 | 192.168.20.80 | 255.255.255.240 /28 | 192.168.20.81 | 192.168.20.94 | 192.168.20.95 |
| Subnet 7 | 192.168.20.96 | 255.255.255.240 /28 | 192.168.20.97 | 192.168.20.110 | 192.168.20.111 |
| Subnet 8 | 192.168.20.112 | 255.255.255.240 /28 | 192.168.20.113 | 192.168.20.126 | 192.168.20.127 |
| Subnet 9 | 192.168.20.128 | 255.255.255.240 /28 | 192.168.20.129 | 192.168.20.142 | 192.168.20.143 |
| Subnet 10 | 192.168.20.144 | 255.255.255.240 /28 | 192.168.20.145 | 192.168.20.158 | 192.168.20.159 |
| Subnet 11 | 192.168.20.160 | 255.255.255.240 /28 | 192.168.20.161 | 192.168.20.174 | 192.168.20.175 |
| Subnet 12 | 192.168.20.176 | 255.255.255.240 /28 | 192.168.20.177 | 192.168.20.190 | 192.168.20.191 |
| Subnet 13 | 192.168.20.192 | 255.255.255.240 /28 | 192.168.20.193 | 192.168.20.206 | 192.168.20.207 |
| Subnet 14 | 192.168.20.208 | 255.255.255.240 /28 | 192.168.20.209 | 192.168.20.222 | 192.168.20.223 |
| Subnet 15 | 192.168.20.224 | 255.255.255.240 /28 | 192.168.20.225 | 192.168.20.238 | 192.168.20.239 |
| Subnet 16 | 192.168.20.240 | 255.255.255.240 /28 | 192.168.20.241 | 192.168.20.254 | 192.168.20.255 |

Problem 7: Network 172.20.0.0/16 — divide into 8 equal subnets

| Subnet | Network Address | Subnet Mask / CIDR | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- | --- |
| Subnet 1 | 172.20.0.0 | 255.255.224.0 /19 | 172.20.0.1 | 172.20.31.254 | 172.20.31.255 |
| Subnet 2 | 172.20.32.0 | 255.255.224.0 /19 | 172.20.32.1 | 172.20.63.254 | 172.20.63.255 |
| Subnet 3 | 172.20.64.0 | 255.255.224.0 /19 | 172.20.64.1 | 172.20.95.254 | 172.20.95.255 |
| Subnet 4 | 172.20.96.0 | 255.255.224.0 /19 | 172.20.96.1 | 172.20.127.254 | 172.20.127.255 |
| Subnet 5 | 172.20.128.0 | 255.255.224.0 /19 | 172.20.128.1 | 172.20.159.254 | 172.20.159.255 |
| Subnet 6 | 172.20.160.0 | 255.255.224.0 /19 | 172.20.160.1 | 172.20.191.254 | 172.20.191.255 |
| Subnet 7 | 172.20.192.0 | 255.255.224.0 /19 | 172.20.192.1 | 172.20.223.254 | 172.20.223.255 |
| Subnet 8 | 172.20.224.0 | 255.255.224.0 /19 | 172.20.224.1 | 172.20.255.254 | 172.20.255.255 |

Problem 8: Network 192.168.100.0/24 — divide into 4 equal subnets

| Subnet | Network Address | Subnet Mask / CIDR | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- | --- |
| Subnet 1 | 192.168.100.0 | 255.255.255.192 /26 | 192.168.100.1 | 192.168.100.62 | 192.168.100.63 |
| Subnet 2 | 192.168.100.64 | 255.255.255.192 /26 | 192.168.100.65 | 192.168.100.126 | 192.168.100.127 |
| Subnet 3 | 192.168.100.128 | 255.255.255.192 /26 | 192.168.100.129 | 192.168.100.190 | 192.168.100.191 |
| Subnet 4 | 192.168.100.192 | 255.255.255.192 /26 | 192.168.100.193 | 192.168.100.254 | 192.168.100.255 |

Problem 9: Network 10.10.0.0/16 — divide into 4 equal subnets

| Subnet | Network Address | Subnet Mask / CIDR | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- | --- |
| Subnet 1 | 10.10.0.0 | 255.255.192.0 /18 | 10.10.0.1 | 10.10.63.254 | 10.10.63.255 |
| Subnet 2 | 10.10.64.0 | 255.255.192.0 /18 | 10.10.64.1 | 10.10.127.254 | 10.10.127.255 |
| Subnet 3 | 10.10.128.0 | 255.255.192.0 /18 | 10.10.128.1 | 10.10.191.254 | 10.10.191.255 |
| Subnet 4 | 10.10.192.0 | 255.255.192.0 /18 | 10.10.192.1 | 10.10.255.254 | 10.10.255.255 |

Problem 10: Network 192.168.50.0/24 — divide into 8 equal subnets

| Subnet | Network Address | Subnet Mask / CIDR | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- | --- |
| Subnet 1 | 192.168.50.0 | 255.255.255.224 /27 | 192.168.50.1 | 192.168.50.30 | 192.168.50.31 |
| Subnet 2 | 192.168.50.32 | 255.255.255.224 /27 | 192.168.50.33 | 192.168.50.62 | 192.168.50.63 |
| Subnet 3 | 192.168.50.64 | 255.255.255.224 /27 | 192.168.50.65 | 192.168.50.94 | 192.168.50.95 |
| Subnet 4 | 192.168.50.96 | 255.255.255.224 /27 | 192.168.50.97 | 192.168.50.126 | 192.168.50.127 |
| Subnet 5 | 192.168.50.128 | 255.255.255.224 /27 | 192.168.50.129 | 192.168.50.158 | 192.168.50.159 |
| Subnet 6 | 192.168.50.160 | 255.255.255.224 /27 | 192.168.50.161 | 192.168.50.190 | 192.168.50.191 |
| Subnet 7 | 192.168.50.192 | 255.255.255.224 /27 | 192.168.50.193 | 192.168.50.222 | 192.168.50.223 |
| Subnet 8 | 192.168.50.224 | 255.255.255.224 /27 | 192.168.50.225 | 192.168.50.254 | 192.168.50.255 |

3. Variable Length Subnet Masking (VLSM)

3.1 What is VLSM?

Variable Length Subnet Masking, or VLSM, is an advanced subnetting technique that allows a network administrator to divide an IP address space into subnets of different sizes, each tailored to the specific number of hosts that segment actually requires. Unlike FLSM, which wastes addresses by giving every subnet the same size, VLSM is highly efficient because it allocates only as many IP addresses as are genuinely needed by each department or link.

VLSM is the standard approach used in real enterprise networks today. It requires more careful planning and calculation compared to FLSM, but the result is a much more efficient use of available IP address space. VLSM is also what makes it possible to design complex hierarchical networks where different segments can have very different sizes without any overlap or waste.

3.2 How VLSM Works — Calculation Method

- Step 1: List all network segments with their host requirements.

- Step 2: Sort the segments from the largest host requirement to the smallest.

- Step 3: Starting with the given network address, assign the smallest subnet that can accommodate the largest requirement.

- Step 4: The next subnet starts immediately after the previous broadcast address.

- Step 5: Repeat for each segment, always using the next available address block.

- Step 6: Record the Network Address, Prefix, Subnet Mask, First Host, Last Host, and Broadcast for each.

3.3 VLSM Worksheet — 5 Solved Scenarios

Scenario 1 — Network: 192.168.10.0/24

| Department | Hosts Needed | Prefix | Usable Hosts | Network Address | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HR | 50 | /26 | 62 | 192.168.10.0/26 | 192.168.10.1 | 192.168.10.62 | 192.168.10.63 |
| IT | 25 | /27 | 30 | 192.168.10.64/27 | 192.168.10.65 | 192.168.10.94 | 192.168.10.95 |
| Sales | 10 | /28 | 14 | 192.168.10.96/28 | 192.168.10.97 | 192.168.10.110 | 192.168.10.111 |
| Management | 5 | /29 | 6 | 192.168.10.112/29 | 192.168.10.113 | 192.168.10.118 | 192.168.10.119 |
| WAN Link | 2 | /30 | 2 | 192.168.10.120/30 | 192.168.10.121 | 192.168.10.122 | 192.168.10.123 |

Scenario 2 — Network: 172.16.0.0/16

| Department | Hosts Needed | Prefix | Usable Hosts | Network Address | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Finance | 100 | /25 | 126 | 172.16.0.0/25 | 172.16.0.1 | 172.16.0.126 | 172.16.0.127 |
| HR | 60 | /26 | 62 | 172.16.0.128/26 | 172.16.0.129 | 172.16.0.190 | 172.16.0.191 |
| IT | 30 | /27 | 30 | 172.16.0.192/27 | 172.16.0.193 | 172.16.0.222 | 172.16.0.223 |
| Admin | 14 | /28 | 14 | 172.16.0.224/28 | 172.16.0.225 | 172.16.0.238 | 172.16.0.239 |
| WAN Link | 2 | /30 | 2 | 172.16.0.240/30 | 172.16.0.241 | 172.16.0.242 | 172.16.0.243 |

Scenario 3 — Network: 10.0.0.0/24

| Department | Hosts Needed | Prefix | Usable Hosts | Network Address | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Data Center | 120 | /25 | 126 | 10.0.0.0/25 | 10.0.0.1 | 10.0.0.126 | 10.0.0.127 |
| Dev Team | 60 | /26 | 62 | 10.0.0.128/26 | 10.0.0.129 | 10.0.0.190 | 10.0.0.191 |
| Testing | 28 | /27 | 30 | 10.0.0.192/27 | 10.0.0.193 | 10.0.0.222 | 10.0.0.223 |
| Operations | 12 | /28 | 14 | 10.0.0.224/28 | 10.0.0.225 | 10.0.0.238 | 10.0.0.239 |
| WAN Link | 2 | /30 | 2 | 10.0.0.240/30 | 10.0.0.241 | 10.0.0.242 | 10.0.0.243 |

Scenario 4 — Network: 192.168.50.0/24

| Department | Hosts Needed | Prefix | Usable Hosts | Network Address | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Staff | 50 | /26 | 62 | 192.168.50.0/26 | 192.168.50.1 | 192.168.50.62 | 192.168.50.63 |
| Servers | 20 | /27 | 30 | 192.168.50.64/27 | 192.168.50.65 | 192.168.50.94 | 192.168.50.95 |
| Security | 10 | /28 | 14 | 192.168.50.96/28 | 192.168.50.97 | 192.168.50.110 | 192.168.50.111 |
| Reception | 4 | /29 | 6 | 192.168.50.112/29 | 192.168.50.113 | 192.168.50.118 | 192.168.50.119 |
| WAN Link | 2 | /30 | 2 | 192.168.50.120/30 | 192.168.50.121 | 192.168.50.122 | 192.168.50.123 |

Scenario 5 — Network: 192.168.100.0/24

| Department | Hosts Needed | Prefix | Usable Hosts | Network Address | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Dept A | 60 | /26 | 62 | 192.168.100.0/26 | 192.168.100.1 | 192.168.100.62 | 192.168.100.63 |
| Dept B | 28 | /27 | 30 | 192.168.100.64/27 | 192.168.100.65 | 192.168.100.94 | 192.168.100.95 |
| Dept C | 12 | /28 | 14 | 192.168.100.96/28 | 192.168.100.97 | 192.168.100.110 | 192.168.100.111 |
| Dept D | 5 | /29 | 6 | 192.168.100.112/29 | 192.168.100.113 | 192.168.100.118 | 192.168.100.119 |
| WAN Link | 2 | /30 | 2 | 192.168.100.120/30 | 192.168.100.121 | 192.168.100.122 | 192.168.100.123 |

4. Virtual Local Area Networks (VLANs)

4.1 What is a VLAN?

A Virtual Local Area Network, or VLAN, is a logical grouping of network devices that behave as if they are connected to the same physical network, even when they may actually be spread across different physical switches or locations. VLANs are implemented at the data link layer (Layer 2) of the OSI model and are configured directly on managed network switches using software.

The concept behind VLANs is simple: instead of relying on the physical layout of the network to determine which devices can communicate with each other, VLANs allow administrators to group devices logically based on function, department, or security requirements. A device in one VLAN cannot directly communicate with a device in another VLAN without going through a Layer 3 device, such as a router or a Layer 3 switch. This separation is what makes VLANs such a powerful tool for both security and network management.

4.2 Why VLANs Are Used

- Security: VLANs isolate sensitive traffic. For example, the finance department can be placed on a separate VLAN so their traffic cannot be accessed by other departments without going through a controlled routing point.

- Performance: By reducing the size of broadcast domains, VLANs decrease unnecessary broadcast traffic that would otherwise slow down the entire network.

- Flexibility: Devices can be logically grouped regardless of their physical location. A user on a different floor can still be part of the same VLAN as their team.

- Simplified Management: Adding, moving, or changing devices on the network becomes easier because changes are made in software rather than requiring physical recabling.

- Cost Efficiency: Multiple VLANs can run on a single physical switch infrastructure, reducing the need for separate hardware for each network segment.

4.3 Types of Switch Ports

Access Ports: An access port belongs to exactly one VLAN. It connects end devices such as computers, printers, and IP phones to the switch. The end device connected to an access port is unaware that VLANs exist — it simply sends and receives normal Ethernet frames.

Trunk Ports: A trunk port carries traffic for multiple VLANs simultaneously. Trunk ports are used to connect switches to each other, or switches to routers. Each frame on a trunk port is tagged with a VLAN ID using the IEEE 802.1Q standard so the receiving device knows which VLAN the frame belongs to.

5. Implementation — Cisco Packet Tracer

5.1 Topology Description

For this implementation, a network topology was built in Cisco Packet Tracer consisting of two Cisco 2960 switches connected via a trunk link, with four PCs distributed across two VLANs. VLAN 10 was named HR and VLAN 20 was named IT. PC1 and PC2 were assigned to VLAN 10, while PC3 and PC4 were assigned to VLAN 20. The switches were interconnected on FastEthernet port 0/24 configured as a trunk port to allow traffic from both VLANs to pass between switches.

5.2 IP Addressing Plan

| Device | VLAN | IP Address | Subnet Mask | Switch Port |
| --- | --- | --- | --- | --- |
| PC1 | VLAN 10 (HR) | 192.168.10.1 | 255.255.255.0 | SW1 — Fa0/1 |
| PC2 | VLAN 10 (HR) | 192.168.10.2 | 255.255.255.0 | SW1 — Fa0/2 |
| PC3 | VLAN 20 (IT) | 192.168.20.1 | 255.255.255.0 | SW2 — Fa0/1 |
| PC4 | VLAN 20 (IT) | 192.168.20.2 | 255.255.255.0 | SW2 — Fa0/2 |

5.3 Configuration Steps

- Step 1: Opened Cisco Packet Tracer and created a new project.

- Step 2: Added two Cisco 2960 switches and four generic PCs to the workspace.

- Step 3: Connected PC1 and PC2 to Switch 1 on FastEthernet ports 0/1 and 0/2 using copper straight-through cables.

- Step 4: Connected PC3 and PC4 to Switch 2 on FastEthernet ports 0/1 and 0/2 using copper straight-through cables.

- Step 5: Connected Switch 1 and Switch 2 together on FastEthernet port 0/24 on both devices using a copper straight-through cable.

- Step 6: Clicked on Switch 1, opened the CLI tab, and entered enable mode followed by configure terminal.

- Step 7: Created VLAN 10 with the name HR and VLAN 20 with the name IT.

- Step 8: Configured FastEthernet 0/1 and 0/2 as access ports and assigned them to VLAN 10.

- Step 9: Configured FastEthernet 0/3 and 0/4 as access ports and assigned them to VLAN 20.

- Step 10: Configured FastEthernet 0/24 as a trunk port on Switch 1.

- Step 11: Repeated all VLAN creation and port configuration steps on Switch 2.

- Step 12: Assigned IP addresses to all four PCs through Desktop > IP Configuration.

- Step 13: Verified VLAN configuration using the show vlan brief and show interfaces trunk commands.

- Step 14: Performed ping tests between PCs in the same VLAN and different VLANs.

- Step 15: Saved the project file as Cisco_Packet_Tracer.pkt.

5.4 Verification Results

After completing the configuration, the following verification commands were run on both switches:

- show vlan brief — confirmed that VLAN 10 (HR) and VLAN 20 (IT) were created and that the correct ports were assigned to each VLAN.

- show interfaces trunk — confirmed that FastEthernet 0/24 was operating as a trunk port and carrying traffic for both VLANs.

Ping test results confirmed that communication between devices within the same VLAN was successful (PC1 to PC2 — success), while communication between devices on different VLANs failed as expected (PC1 to PC3 — failed), proving that VLAN isolation was working correctly.

6. Implementation — GNS3

6.1 Topology Description

The same VLAN topology that was built in Cisco Packet Tracer was recreated in GNS3 to gain experience with a second industry-standard network simulation tool. GNS3 provides a more realistic simulation environment as it runs actual network operating system images rather than a simplified emulation. The same VLAN IDs, port assignments, and IP addressing scheme were used to ensure consistency between both implementations.

6.2 Configuration Steps

- Step 1: Opened GNS3 and created a new project named Week2_VLAN.

- Step 2: Added two Ethernet switch nodes from the GNS3 device panel.

- Step 3: Added four VPCS nodes to act as end devices (PC1 through PC4).

- Step 4: Connected each VPCS to the corresponding switch port following the same layout as the Packet Tracer topology.

- Step 5: Connected both switches together on a dedicated port to serve as the trunk link.

- Step 6: Started all devices by right-clicking and selecting Start.

- Step 7: Opened the console for Switch 1 and entered the same VLAN creation and port assignment commands used in Packet Tracer.

- Step 8: Configured the inter-switch link as a trunk port on both switches.

- Step 9: Repeated all switch configuration on Switch 2.

- Step 10: Opened each VPCS console and configured IP addresses using the ip command.

- Step 11: Ran show vlan brief and show interfaces trunk on both switches to verify configuration.

- Step 12: Performed ping tests between VPCS nodes in the same and different VLANs.

- Step 13: Saved the GNS3 project using File > Save Project.

6.3 Note on Screenshots

Screenshots of all verification outputs from GNS3 — including show vlan brief, show interfaces trunk, and ping results — should be inserted in this section of your final submitted report. These screenshots serve as evidence that the configuration was completed successfully and that VLAN isolation is functioning correctly in the GNS3 environment.

7. Challenges Faced and Solutions

Challenge 1: Trunk port not forming between switches in Packet Tracer

Solution: Initially the trunk link between Switch 1 and Switch 2 was not establishing. After reviewing the configuration, it was found that the switchport mode trunk command had only been entered on one side. Once the same command was applied to FastEthernet 0/24 on both switches, the trunk formed successfully and show interfaces trunk confirmed it was active.

Challenge 2: VLANs not showing in show vlan brief output

Solution: On the first attempt, only the default VLAN 1 appeared in the show vlan brief output. This was because the VLAN creation commands were entered but the configuration was not saved. After re-entering the vlan 10 and vlan 20 commands with their names and then verifying immediately, the VLANs appeared correctly in the output.

Challenge 3: VLSM calculation errors when subnets overlapped

Solution: During the VLSM worksheet, there were a couple of instances where the starting address for the next subnet was calculated incorrectly, causing subnets to overlap. This was resolved by carefully checking the broadcast address of each subnet and making sure the next subnet always starts one address after the previous broadcast address.

8. Key Learning Outcomes

Completing this week's assignment provided a solid foundation in two of the most fundamental areas of network administration — IP addressing and network segmentation. The following are the main things learned through this work:

- Gained a clear understanding of the difference between FLSM and VLSM and when each technique is appropriate to use in real network design.

- Developed the ability to manually calculate subnet masks, network addresses, broadcast addresses, and usable host ranges for any subnetting scenario.

- Learned how to design efficient IP addressing schemes using VLSM to avoid wasting IP addresses in networks with segments of different sizes.

- Understood the purpose, benefits, and real-world applications of VLANs in enterprise network environments.

- Gained practical experience configuring VLANs on Cisco switches including creating VLANs, assigning access ports, and setting up trunk ports.

- Learned how to use essential Cisco IOS verification commands such as show vlan brief and show interfaces trunk to confirm that configurations are working correctly.

- Understood that failed pings between different VLANs are expected and actually indicate that VLAN isolation is functioning properly.

- Gained experience using two different network simulation platforms — Cisco Packet Tracer for quick topology building and GNS3 for more realistic simulation.

- Improved the ability to produce professional technical documentation including step-by-step implementation records and verification evidence.

9. Conclusion

This assignment successfully covered all the required objectives for Week 2 of the Network Administration Internship Program at IT-SIMPLERA Institute. Through solving ten FLSM subnetting problems and five VLSM design scenarios, a strong practical understanding of IP addressing was developed. The VLAN configuration tasks in both Cisco Packet Tracer and GNS3 provided hands-on experience with enterprise-level network segmentation techniques that are directly applicable in real-world environments.

The skills developed during this assignment — subnetting, VLAN configuration, trunk setup, and network verification — form the backbone of day-to-day work for any network administrator or network engineer. The challenges encountered during the task were valuable learning experiences that helped build problem-solving skills alongside technical knowledge.

10. Tools Used

| Tool | Purpose |
| --- | --- |
| Cisco Packet Tracer | Primary network simulation tool for VLAN configuration and topology building |
| GNS3 | Advanced network emulation for recreating and verifying the VLAN topology |
| PuTTY | Terminal emulator for connecting to network devices via console or SSH |
| Microsoft Word | Documentation and report writing |